Quantification of Lentiviral Integrating Units by Real Time PCR

| Standard Operating Procedure Vector Core LSU Medical Center New Orleans | Effective Date: R.K. 5-13-03 | Lentivirus Testing Protocol |
| --- | --- | --- |
|  | Revised Date: RK 9-26-03 | Protocol #: LTP-004-000 |
|  | Written by / Date: R.K. 5-13-03 | Performed by / Date: |
|  | Approved by / Date: | Reviewed by / Date: |

Quantification of Lentiviral Integrating Units by Real Time PCR

- PURPOSE

  - To titer for integrating units in lentivirus stocks.

- MATERIALS

  - General tissue culture supplies including Aerosol Resistant Tips

  - Suitable cell line for tittering lentivirus like HOS, 239T or HeLa

  - DNaseI 5.0 U/μl (Takara)

  - 25 mM MgCl2

  - Polybrene 8μg/μl

  - .25% Trypsin / 1mM EDTA (Gibco)

  - PBS

  - PBS w/10% FBS

  - DMEM Glu  10% FBS 1% ps/lg

  - DNeasy Tissue Kit (Qiagen 69506)

  - 10ng/μl tRNA

  - Forward primer

    - 5’ TTC GCA GTT AAT CCT GGC CTT 3’

  - Reverse primer

    - 5’ GCA CAC AAT AGA GGA CTG CTA TTG TA 3’

  - Probe

    - 5’ /56-FAM/TAG AGA CAT CAG AAG GCT GTA GAC AAA /36-TAMTph/ 3’

  - TaqMan Universal 2x PCR mix (PE-4304437)

  - RNaseP Detection Kit

  - Vector plasmid that contains target gene for Standard

  - 1 ml Sterile Water

  - Optic 96 well plate

  - Optic well covers

  - ABI 7700 Sequence Detector w/ SDS software

- METHOD

  - Day 1

    - Seed 1 x 105  cells per well of a 6 well plate for same day transduction

      - Alternativly 5 x 104 cells / well can be seeded the prior day

    - Place 100μl of virus stock into a new tube and add .5l DNaseI and      12l of  25 mM MgCl2

    - Incubate at room temperature  for 15 minutes

      - During this time aspirate old media from each well and add 400l DMEM with .5l of polybrene

        - Making a 25 ml working solution of DMEM with 31l of polybrene is sufficient for 50+ wells

    - Transfer all 112.5l virus/DNase to a respective well

    - Be sure to mock transduce at least one well for control

    - Incubate at 37° C and 5% CO2 for 16-20 hours

  - Day 2

    - Aspirate transduction media from plate and add a fresh 2 ml DMEM per well

    - Allow 48 hours of incubation at 37° C and 5% CO2 for integration

  - Day 4

    - Aspirate growth media from each well

    - Add 500l of Trypsin/EDTA to each well

    - Incubate at 37° C and 5% CO2 until cells round and detach

    - Stop the trypsinization by adding 500l of PBS w/10% FBS

    - Harvest cells into a 2 ml eppendorf tube

      - The harvest is best done with a P1000 and Aerosol Tips

    - Spin out cells at 500g for 5 min

    - Aspirate supernatant and resuspend cell pellet in 200l of PBS

    - Proceed to DNeasy tissue kit protocol after reading below

      - When extracting genomic DNA it is important to do the following:

        - Have a set of dedicated pipettes just for genomic isolation

        - Clean pipettes regularly

        - Do it in a location that has NO other work with plasmid DNA

        - Aliquot as much of the kit as possible to reduce carry over i.e. RNase

        - Make final elution in 200l of AE buffer

    - Prepare RNaseP standard as per AB protocol

    - Prepare standard solutions as follows in Eppendorf tubes starting with pNLCMVEGFP   28.5 g / ml =2.8 x 1010  copies / l

| Dilution | Volume of standard | Volume of tRNA 10 ng/ml | Concentration | Copy number |
| --- | --- | --- | --- | --- |
| 2.8 x 1010 / l (A) |  |  |  | Not used |
| B | 10 l of A | 130 l | 1 x 109 / 5 l | 109 |
| C | 10 l of B | 90 l | 1 x 108 / 5 l | 108 |
| D | 10 l of C | 90 l | 1 x 107 / 5 l | 107 |
| E | 10 l of D | 90 l | 1 x 106 / 5 l | 106 |
| F | 10 l of E | 90 l | 1 x 105 / 5 l | 105 |
| G | 10 l of F | 90 l | 1 x 104 / 5 l | 104 |
| H | 10 l of G | 90 l | 1 x 103 / 5 l | 103 |
| I | 10 l of H | 90 l | 1 x 102 / 5 l | 102 |

    - UV sterilize as much equipment as possible

    - Prepare Master Mix For 40 reactions(rx):

      - 900l  2x TaqMan Master mix

      - 4 l of 100pm/l Forward primer; 10 pm/rx

      - 4 l of 100pm/l Reverse primer; 10 pm/rx

      - 4 l of 100pm/l Probe; 10 pm/rx

      - 888 l Sterile Water

    - Set up Optic 96 well plate for appropriate number of reactions

      - Duplicate standards and samples

      - Only one negative control needed

    - FIRST add 45l Master Mix to each needed well

      - No need to change tips here.

    - SECOND add 5l of standard or sample to each respective well

      - CHANGE PIPETTE TIPS EACH TIME.

    - Cover with Optic well covers by roller or other device DO NOT TOUCH

    - Place in ABI7700 and run according to protocol